© David Cole , Dr. H. K. D. H. Bhadeshia,

University of Cambridge, July, 1999

This file contains the steps to run the creep rupture strength model.

- Copy files onto hard disk, without disturbing the structure and remove the “READ-ONLY” status for files ‘test.dat’ and ‘Result.dat’.

- Edit the test.dat file to set input variables. The input variables are in row;

Column Variable

- log(creep rupture time / h)

- Temperature (K)

- C (wt%)

- Si

- Mn

- P

- S

- Cr

- Mo

- W

- Ni

- Cu

- V

- Nb

- N

- Al

- B

- Co

- Ta

- O

- Normalising Temperature (K)

- Normalising Time (h)

- Cooling rate of Normalise in furnace (0 or 1)

- Cooling rate of Normalise in air (0 or 1)

- Cooling rate of Normalise for oil quench (0 or 1)

- Cooling rate of Normalise for water quench (0 or 1)

- Tempering Temperature (K)

- Tempering Time (h)

- Cooling rate of Temper in furnace (0 or 1)

- Cooling rate of Temper in air (0 or 1)

- Cooling rate of Temper for oil quench (0 or 1)

- Cooling rate of Temper for water quench (0 or 1)

- Annealing Temperature (K)

- Annealing Time (h)

- Cooling rate of Annealing in furnace (0 or 1)

- Cooling rate of Annealing in air (0 or 1)

- Double click on “creep’' icon , to run program.

- Results are stored in a file called 'Result.dat'. The format of this file is;

Creep Rupture Stress / Mpa       Lower error bound / Mpa       Upper error bound / MPa

End of task.