© Sree Harsha Lalam , Dr. H. K. D. H. Bhadeshia,

University of Cambridge, July, 1999

This file contains the steps to run Yield Strength Model.

- Copy files onto hard disk, without disturbing the structure and remove the “READ-ONLY” status for files ‘test.dat’ and ‘YSresult.dat’.

- Edit the test.dat file to set input variables. The input variables are in row;

Column Variable

1	C

2	Si

3	Mn

4	S

5	P

6	Ni

7	Cr

8	Mo

9	V

10	Cu

11	Co

12	W

13	Ti(p)

14	B(p)

15	Nb(p)

16	HI

17	IT

18	pwhtT

19	pwhtt

p - PPM

HI - Heat Input in KJ/mm

IT- Interpass Temperature in deg.C

pwhtT - post weld heat treatment  temperature in deg. C

pwhtt - post weld heat treatment time in hr.

Note: For as-welded calculations:

Pwht temperature = 20 deg. C and   Pwht time = 0.0

- Double click on “YS’' icon , to run program.

- Results are stored in a file called 'YSresult.dat'. The format of the file is;

Yield Strength (MPa)     Error Bar (MPa)      YS – Error Bar       YS + Error Bar

End of task.