© Sree Harsha Lalam , Dr. H. K. D. H. Bhadeshia,

University of Cambridge, July, 1999

This file contains the steps to run Charpy Toughness Model.

- Copy files onto hard disk, without disturbing the structure and remove the “READ-ONLY” status for files ‘test.dat’ and ‘Chpresult.dat’.

- Edit the test.dat file to set input variables. The input variables are in row;

Column Variable

1	C

2	Si

3	Mn

4	S

5	P

6	Ni

7	Cr

8	Mo

9	V

10	Cu

11	Co

12	W

13	Oxygen (p)

14	Ti(p)

15	N (p)

16	B(p)

17	Nb(p)

18	HI

19	IT

20	pwhtT

21	pwhtt

22	Test Temperature (deg. C)

p – PPM wt.

HI - Heat Input in KJ/mm

IT- Interpass Temperature in deg.C

pwhtT - post weld heat treatment temperature in deg. C

pwhtt - post weld heat treatment time in hr.

Note: For as-welded calculations:

Pwht temperature = 20 deg. C and   Pwht time = 0.0

- Double click on “Chp’' icon , to run program.

- Results are stored in a file called 'Chpresult.dat'. The format of this file is;

- Toughness (J)      Error Bar (J)     Toughness – Error  Bar    Toughness + Error Bar

End of task.