© Sree Harsha Lalam and Dr.  H K D H Bhadeshia

University of Cambridge

June, 1999

1) Copy all the files into your hard disk..

To use the program ;

- Enter your data in order in file “test.dat” as indicated in 3.   Enter number of data lines in  the file “no_of_rows.dat” . Then type UTS.EXE'  at command prompt or in windows double click on the ‘uts.exe’ icon.

- The results are written in Uresult. Format of Uresult;

- Predicted UTS  Error Bar   Predicted – Error   Predicted + Error

(MPa)		(MPa)		(MPa)		  (MPa)

2) In the input file is “test.dat”, total number of input varibles are 20.

3) The input variables, set in a row of the input file, are :

C weight %

Si weight %

Mn weight %

S weight %

P weight %

Ni weight %

Cr weight %

Mo weight %

V weight %

Cu weight %

Co weight %

W weight %

O ppmw

Ti ppmw

B ppmw

Nb ppmw

Heat input kJ/mm

Interpass Temperature( C)

PWHT temperature  (C) [ If as-welded, set temp=20, time=0]

PWHT time hours

4) The “S” directory contains the source files and the executables required to normalise and un-normalise the data and output.

5) The “C” directory contains the weight files.

6) The “d” contains the normalised target valuses generated during training.

7) The outputs are written in “Uresult” file.

Example INPUT file

0.0 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.0200 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.0400 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.0600 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.0800 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.1000 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.12 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.14 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.1600 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

0.1800 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 350 18 10 200 1.3 300 20 0.0

Example RESULT file

743.7818796    103.5349718    640.2469078    847.3169320

774.8372208     93.8765126    680.9607082    868.7137334

805.1369370     89.2066292    715.9302272    894.3435662

834.4896032     90.0721926    744.4174106    924.5617958

862.7131440     96.3637480    766.3493960    959.0768920

889.6428130    107.4081242    782.2346888    997.0509372

915.1395752    122.3790100    792.7604846   1037.5185852

939.0970386    140.6004132    798.4967060   1079.6974518

961.4488694    161.6184752    799.8303942   1123.0673446

982.1743534    185.1536752    797.0206782   1167.3280286