© Sree Harsha Lalam and Dr.  H K D H Bhadeshia

University of Cambridge

June, 1999

1) Copy all the files into your hard disk..

To use the program ;

- Enter your data in order in file “test.dat” as indicated in 3.   Enter number of data lines in  the file “no_of_rows.dat” . Then type YS.EXE'  at command prompt or in windows double click on the ‘ys.exe’ icon.

- The results are written in Yresult. Format of Yresult;

Predicted YS  Error Bar   Predicted – Error   Predicted + Error

(MPa)		(MPa)		(MPa)		  (MPa)

2) In the input file is “test.dat”, total number of input variables are 19.

3) The input variables, set in a row of the input file, are :

C weight %

Si weight %

Mn weight %

S weight %

P weight %

Ni weight %

Cr weight %

Mo weight %

V weight %

Cu weight %

Co weight %

W weight %

Ti ppmw

B ppmw

Nb ppmw

Heat input kJ/mm

Interpass Temperature( C)

PWHT temperature  (C) [ If as-welded, set temp=20, time=0]

PWHT time hours

4) The “S” directory contains the source files and the executables required to normalise and un-normalise the data and output.

5) The “C” directory contains the weight files.

6) The “d” contains the normalised target valuses generated during training.

7) The outputs are written in “Yresult” file.

Example INPUT file

0.0000 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.0200 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.0400 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.0600 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.0800 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.1000 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.1200 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.1400 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.1600 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

0.1800 0.27 0.6 0.007 0.01 0.05 2.22 0.3 0.2 0.0133 0.0 0.5 18 10 200 1.3 300.0 20 0.0

Example RESULT file

706.2426072    122.9664456    583.2761616    829.2091266

716.2475256    113.1753258    603.0721998    829.4228514

727.4495538    106.1407098    621.3089178    833.5902636

739.8266256    101.1055572    638.7210684    840.9321090

753.3037602     97.5769578    655.7267286    850.8807180

767.7704790     95.4148392    672.3556398    863.1853920

783.0973368     94.7516724    688.3457382    877.8490092

799.1499438     95.8233960    703.3264740    894.9733398

815.8006260     98.7969456    717.0036804    914.5975716

832.9363956    103.6668600    729.2695356    936.6032556